SNMP Changes.

Author: Akshata Jadhav

Designation: CIS Automation Developer

Date: 9 August 2019

REVISION HISTORY

| VERSION | DATE | Summary Of Changes | AUTHOR | REVIEWED BY | RIVISION REMARKS (YES / NO) |
| --- | --- | --- | --- | --- | --- |
| 0.1 | 31st May, 2019 | Initial Draft | Akshata Jadhav |  |  |

Contents

Introduction	4

Flow Chart	4

Process	5

Execution Steps	5

Input Parameter:	5

Output	5

Scope	5

Prerequisites	5

Deployment Steps / Procedure	6

# Introduction

The script will connect to device and execute commands to add snmp changes and then check the snmp status executing the command “show run | i snmp”.

# Flow Chart

# Process

# Execution Steps

- Script Logins to switch and Adds snmp changes with set of commands from commands.txt file.

- The set of  commands  triggered are as follows #Config t

  - # snmp-server community ICH-Live01 RO 10

  - # snmp-server community 1nP3x RO 5

  - # snmp-serever trap-source Vlan207

  - # snmp-server location LNG Onshore

  - # snmp-server contact INPEX Service Desk

  - # snmp-server enable traps tty

  - # snmp-sercfsfver enable traps license

  - # snmp-server enable traps config

  - # snmp-server enable traps syslog

  - # snmp-server enable traps envmon fan shutdown supply temperature status

  - # snmp-server host XX.XX.XX.XX version 2c 1CH-Liv301

  - # snmp-server host XX.XX.XX.XX 1nP3x

  - # snmp-server host XX.XX.XX.XX version 2c ICH-Live01

  - # snmp ifmib ifindex persist

  - # wr

  - # end

- Get the status of the SNMP changes added using command “show run | i snmp”

# Input Parameter:

# Input required for SNMP_changes

Input Files:-

- data.csv :- Device IP, Login ID, Encrypted Password

Sample input:

# Output

# SNMP status is provided in text file in output folder

# Scope

This script will work on python 3.4 and above version.

# Prerequisites

•Python 3.4 and above version

# Deployment Steps / Procedure

- Edit the input file with device credentials.

- Run the script then the output text file is generated.